Human-Informed Topology Optimization 2.0 (HiTop 2.0)

HiTop is a human in the loop design optimization code first published in the paper "Human Informed Topology Optimization: Interactive Application of Feature Size Controls" (Ha, D. and Carstensen, J.V., 2023, Structural and Multidisciplinary Optimization 66:59. DOI 10.1007/s00158-023-03512-0).

HiTop 2.0 builds upon the original version of HiTop in the recent paper:

Gillian Schiffer, Dat Quoc Ha & Josephine V. Carstensen (2023) HiTop 2.0: combining topology optimisation with multiple feature size controls and human preferences, Virtual and Physical Prototyping, 18:1, DOI: 10.1080/17452759.2023.2268603.

Different from the original HiTop, HiTop 2.0 implements a GUI that allows for a flexible design problem formulation. Additionally, the user can make 1 or 2 changes to the minimum solid, minimum void, maximum solid, and/or passive regions of interest in the design.

File Familiarization:

To start, users should download the HiTop2_0 folder. Enter the folder and you will find three files.

- Parameters.m – this is where the user specifies the parameters below. The user will also find the authors’ general guidelines on what range for the parameters they have found to work best.

| Nelx | Number of elements in the x-direction | Nely | Number of elements in the y-direction |
| --- | --- | --- | --- |
| Volfrac | Volume fraction for optimization problem | rminS | Radius of minimum solid feature size |
| rminV | Radius of minimum void feature size | betaMMA | beta value for the Heavisides projection |
| betaHP | Same as betaMMA | Penal | Penalization factor for the SIMP method |

- hiTopDriver.m – this is the driver file to run the HiTop 2.0 program.

- addpath folder – this folder contains the supporting MATLAB scripts and MMA folder

How it works (with L-Bracket example):

- Enter the appropriate variables for your design problem in the parameters.m file

- Run the HiTopDriver.m MATLAB script

- Highlight location of force

  - The authors recommend zooming into the mesh to ensure accurate selection.

  - For best results, only select <10 nodes and ensure the node locations are fully enclosed by your drawn rectangle Region of Interest (ROI)

Proper selection of 8 nodes for the 		    Too many nodes selected for load

L-Bracket Problem

  - Double click the ROI

- Specify the direction of the negative force with 1 = x, 2 = y, 3 = x and y

  - Assumed positive x and y directions below

  - For L-Bracket, enter 2

- Enter whether you are going to emplace 1 or 2 boundary conditions

  - For L-Bracket enter 1

- Enter whether you want to manually draw (1) or automatically pick an edge (2) of the design domain for your boundary condition

  - For L-Bracket enter 1

- High the location of the boundary condition

  - Ensure your ROI only encloses the desired nodes

Proper selection of single 	     Too many nodes selected		No nodes are selected

line of nodes on upper

edge of domain

  - Double click the ROI

- Specify the direction of the translation restriction with 1 = x, 2 = y, 3 = x and y

  - For L-Bracket, enter 3

- Enter whether you’d like to specify a solid (1), void (0), or no (0) passive region in the design problem

  - For L-Bracket enter 0

- Draw the passive region

  - The authors find that drawing the passive region zoomed out, then zooming in to refine is a successful method to specify the passive ROI

  - Next, zoom into the areas of the passive ROI that may overlap with force or boundary condition ROIs

For the L-Bracket problem, the 	            Proper selection of nodes 	Passive ROI overlaps location

passive	region should be flush		    for the passive ROI	 		of force nodes

to the force ROI

- 50 Iterations will run automatically

  - For the L-Bracket, the 50 iteration output will look like the the figures below, where a stress plot is presented to the user

- Enter whether you want to make 1 or 2 changes

  - For this example, the user will make 2 changes

- Enter whether you want to specify a new passive ROI (3), max solid ROI (2), min solid (1), or min void (0)?

  - For this example, the user will specify a passive void region (3) at the sharp corner in order to reduce the stress concentration.

- Draw the 1st change ROI

  - For the L-Bracket example:

- Enter which phase you’d like to specify as passive (1) for solid, (0) for void

- 50 Iterations run automatically

  - If the user decided to only make one change, then the algorithm would run to 2000 iterations or convergence

- Enter whether you want to specify a new passive ROI (3), max solid ROI (2), min solid (1), or min void (0)?

  - For this example, the user’s second change will specify a minimum solid length scale in the thin lower, right portion of the L-Bracket.

- Highlight the ROI on the stress plot.

  - The authors have found success when singular members are identified, rather than middle of members.

- Enter contour lines, lambda (degree of gradation), and new radius

  - For this example, the user set contour = 2, radius = 4, and lambda = 5

- Run until convergence.

Final output